DIGITAL PANEL METERS ICO 51

Electrical Measurements 10 000 Points

The function of measurement and range, as well as put on a specific scale are programmable.

The parameters thus configured are protected against all inopportune modifications

- Format 72 x 24 mm

- Function, ranges and scale

- programmable

- Configurable on site

Functions

Measure...............................................

The ICO 51 allow upto measurements of A.C/D.C 400 V (true effective value with or without component continuous health) and measurements of A.C/D.C current (up to 5 A) using small shunt screwed on the back connector block.

Automatic ranging...............................................

This function allows to change automatically the range in same measuring unit.

Hold the measurement...............................................

It is carried out by a simple short-circuit on the back connector block.

Scaling...............................................

Whatever the function, the user can program a scaling.

The value to be posted is entry using the two keys of the keyboard while the value of horn measurement corresponding can be is introduced even way, is measured by the instrument.

(1) Extended from measurement : of - 40 to + 110 % gauge.

(2) ± (% reading + a number of units) on 1 year.

Continuous: of - 40 to 100 % gauge.

Into alternate : of 4 to 100 % gauge from 25 to 500 Hz without D.C component (possible measurements up to 10 Khz on some ranges with less precision).

(3)Current measurement with external shunt.

General characteristics

Display...............................................

From - 1999 to 9999 with indication, LED red, yellow or green,14 mm.

Speed of measurement : 2,5 measures/s.

Temperature coefficient ≤ 10 % préci - sion/°C.

Acceptable maximum voltage on the ranges voltage: 600V.

Common mode...............................................

Acceptable maximum volatge: 250 V~.

Rejection ≥110 dB to 50 Hz on the gauge 500mV continuous..

Climatic environnement...............................................

Nominal field : 0 à 50°C, of 20 to 80% HR without condensation.

Isolated power supply...............................................

230 V~ (± 10 %) 50 à 60 Hz.

Others power supply:

5 V- ± 10 %,

9 à 18 V-

18 à 36 V-,

36 à 72 V-,

24 V~ ± 10 %,

48 V~, ± 10 %,

115 V~± 10 %.

Presentation...............................................

Case ABS.

Model 72 X 24: sealing IP 40.

Weight: approximately 300 G.

Connection...............................................

Removable screwing connectors supplied with instrument.

Dimensions

Options and accessories

External shunts...............................................

Modules screwed on the connector.

AN 8003 : 1 A, 100 mV, accuracy 0,2 %

AN 8004 : 5 A, 100 mV, accuracy 0,5 % .

Dimensions : 25 x 27,5 x 14,8 mm.

Transformers of current...............................................

Classify 0,5.

AN 5844 : 2/1 A,

AN 5845 : 20/1 A,

AN 5846 : 200/1 A.

Metal case seals...............................................

Seal opposite before (IP 65), format 99 x 36 x prof. 120 mm.

Mechanical flange of adaptation...............................................

It makes it possible to adapt the instrument to a drilling of panel of higher size of a few millimetres.

External dimensions : 80 x 32 mm.

Instructions to order

Options and accessories...............................................

Shunt 1 A                         AN 8003

Shunt 5 A                         AN 8004

Transformer of current 2/1 A          AN 5844

Transformer of current 20/1 A         AN 5845

Transformer of current 200/1A         AN 5846

Flange of adaptation                 AMD001

Indicator 12 ways See note            BCM 012