FCC-3 Tuned Dipole Antenna Data

More Information

- Typical Antenna Factor Plot - 3m

Innovation, Quality, and Performance

- Typical Antenna Factor Plot - 10m

Innovation, Quality, and Performance

- Typical VSWR

Innovation, Quality, and Performance

- Numeric Data

FCC-3 Tuned Dipole Antenna Data

3 Meter | 10 Meter
Antenna Page | VSWR Graph | AF Graph

| 3 Meter Antenna Factor |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | Element Length (L/2-in.) | Antenna Factor (dB/m) | Gain (dBi) |
| 170 | 16 1/8 | 12.9 | 2.0 |
| 180 | 15 3/16 | 13.6 | 1.8 |
| 190 | 14 5/16 | 14 | 1.8 |
| 200 | 13 5/8 | 14.8 | 1.5 |
| 210 | 12 7/8 | 15.1 | 1.6 |
| 220 | 12 1/4 | 15.3 | 1.8 |
| 230 | 11 9/16 | 15.6 | 1.9 |
| 240 | 11 1/8 | 15.8 | 2.1 |
| 250 | 10 5/8 | 16.2 | 2.0 |
| 260 | 10 1/4 | 16.6 | 1.9 |
| 270 | 9 15/16 | 16.9 | 2.0 |
| 280 | 9 11/16 | 17.3 | 1.9 |
| 290 | 9 3/16 | 17.6 | 1.9 |
| 300 | 8 3/4 | 17.9 | 1.9 |
| 310 | 8 5/16 | 18.1 | 2.0 |
| 320 | 8 | 18.7 | 1.6 |
| 330 | 7 13/16 | 18.9 | 1.7 |
| 340 | 7 11/16 | 19.1 | 1.8 |

| 10 Meter Antenna Factor |  |  |  |
| --- | --- | --- | --- |
| Frequency (MHz) | Element Length (L/2-in.) | Antenna Factor (dB/m) | Gain (dBi) |
| 170 | 16 1/8 | 12.9 | 2.0 |
| 180 | 15 3/16 | 13.5 | 1.9 |
| 190 | 14 5/16 | 14.1 | 1.7 |
| 200 | 13 5/8 | 14.9 | 1.4 |
| 210 | 12 7/8 | 15.3 | 1.4 |
| 220 | 12 1/4 | 15.4 | 1.7 |
| 230 | 11 9/16 | 15.7 | 1.8 |
| 240 | 11 1/8 | 16 | 1.9 |
| 250 | 10 5/8 | 16.7 | 1.5 |
| 260 | 10 1/4 | 16.9 | 1.6 |
| 270 | 9 15/16 | 17.5 | 1.4 |
| 280 | 9 11/16 | 17.9 | 1.3 |
| 290 | 9 3/16 | 17.6 | 1.9 |
| 300 | 8 3/4 | 18 | 1.8 |
| 310 | 8 5/16 | 18.8 | 1.3 |
| 320 | 8 | 19 | 1.3 |
| 330 | 7 13/16 | 19.5 | 1.1 |
| 340 | 7 11/16 | 19.5 | 1.4 |